# Guidance on CBA tool

# Methodology

## Evaluation methods: cost-benefit analysis (CBA) and social cost-benefit analysis (SCBA)

The Cost Benefit Analysis (CBA) is a practical way of evaluating the desirability of projects. CBA is applicable when two aspects are important: (a) evaluating from long-term perspectives to consider the consequences in the near and far future, (b) a wide view to consider different side-effects on various target groups, including the citizens and private parties. In a nutshell, CBA is a decision support tool, which is applied to enumerate and evaluate all the relevant costs and benefits for the investment plans, policies, or other actions planned by public or private entities.

The overall objectives of CBA for the economy and society are to:

•	Evaluate the impacts of the investment,

•	Justify the feasibility of implementing a specific investment (yes/no decision), and

•	Select the suitable choice, maximise the net benefits, among different Scenarios.

The following steps are usually conducted for CBA:

Figure 1: Six steps for performing a CBA

Two different types of Cost-Benefit Analyses can be conducted, namely:

Table 1: Types of CBAs and their definitions

| Types of CBA | Definition |
| --- | --- |
| Original CBA | A cost-benefit analysis (CBA) is conducted to measure the benefits of a decision or taking action minus the costs associated with taking that action. CBA includes measurable financial metrics such as revenue or costs due to the decision to pursue a project. In this research, the term economically feasible or not feasible are also used to explain the results of the CBA analysis. The economic impacts are identified, and the evaluation is performed taking into consideration the impacts on the national economy. The analysis is based on changes in the market prices. |
| Social CBA | SCBA includes all the factors of CBA analysis. In addition to this, the environmental and social impacts are identified, and the evaluation is performed taking into consideration the impacts on the whole society. The analysis includes both the market prices and the external costs and benefits triggered by the examined investments. |

## CBA methodology

The CBA aims to examine the social profitability of the examined energy efficiency projects, justifying the potential utilization of specific financial schemes (such as subsidies) in relation with the results of the financial analysis. The conduction of the financial analysis and the social CBA leads to the calculation of two different indicators, namely the financial NPV (or the financial IRR) and the social NPV (or the social IRR respectively).

Table 2: The results of financial and social NPVs and the decisions to move forward/to stop

| Potential result of calculations | Financial NPV | Social NPV | Conclusions | Next steps |
| --- | --- | --- | --- | --- |
| 1st | ≥ 0 | ≥ 0 | Economically and socially beneficial to invest/implement projects/interventions. | Promoting these types of investments/interventions |
| 2nd | ≤ 0 | ≥ 0 | Economically is not beneficial. However, socially is important to invest/implement the project/intervention. | Providing financial supports, e.g., subsidies, increasing municipality’s equity share in the investment |
| 3rd | ≥ 0 | ≤ 0 | Economically is beneficial to invest on a project and not socially beneficial. | Penalty for the investors and imposing the incentives that distribute the economic benefits among different groups, such as tax. |
| 4rth | ≤ 0 | ≤ 0 | Economically and socially is not beneficial. | Stop the investments/interventions |

The proposed steps (Figure 1) are explained below for the application of CBA and social CBA on comparing the supply investments to the demand-side investments of improving the energy efficiency of the buildings.

### Step 1: define the system boundaries

In the current tool, the CBA boundaries can be set either at national or regional level for households to depict the effect of the proposed interventions to the society and economy.

### Step 2: define the baseline Scenario

The baseline Scenario is the investment on the supply side for the expansion of fossil fuels and the current energy efficient technologies in the buildings.

### Step 3: specify the alternative Scenarios

The Scenarios 1 to 4 are the alternative Scenarios. The energy efficient equipment/technologies are assessed within the alternative Scenario and the existing equipment/technology within the baseline Scenario.

### Step 4: Identify and quantify the positive and negative externalities

To this context the quantification of either positive/negative externalities is performed to integrate into the analysis various social and environmental criteria, which affect the social welfare.

#### External costs and benefits

External costs/negative externality. A cost that a transaction or activity imposes on a party, who is not part of the transaction or activity. Specifically, it is the cost or benefit that affects a party, who did not choose to incur that cost or benefit. For instance, electricity generation creates pollution that imposes costs on citizens, or the energy consumed in a building or in a car creates pollution that imposes costs on citizens. This cost is not included in the price of energy; thus, it is an external cost that should be taken into consideration when examining the social effectiveness of an investment.

External benefits. Energy efficiency investments aim first at reducing energy consumption, but they have impact also to other challenges such as energy supply security, climate change, employment. In addition, the implementation of energy efficiency interventions can have other “non-energy”, socio-economic and environmental effects such as effects on social welfare or reduced pollution levels.  Four different types of externalities were assessed within the framework of the current study (Figure 2).

Figure 2: Selected types of externalities within the context of the CBA

The methodological approach, which was utilized for each type of externality separately, is described briefly in Table 3.

Table 3: Types of externalities and their descriptions (References: Alberici et al.  and 2014, Russell et al., 2015)

| Categories | Description |
| --- | --- |
| Environmental and health externalities | The environmental and health externalities are differentiated for the various forms of energy carriers and technologies according to the pollutants emitted within the whole life cycle. Various studies attempted to estimate the environmental and health externalities of the RES and energy efficiency technologies. The calculation of the environmental and health costs and benefits was carried out considering the technical characteristics of the implemented interventions, such as, final energy consumption, insulation levels for each building separately. The data per type of technology is provided in Table 4. |
| Macroeconomic effects | The quantification of the macroeconomic effects was performed considering the results of a study, which was conducted by Cambridge Econometrics to provide estimates of the multiple benefits of energy savings for the European citizens and the economy that are associated with different levels of the 2030 EU energy efficiency target. Coefficients were estimated linking the reduction in energy consumption (in Mtoe) to each of the delivered benefits. It should be noted that the coefficients represent average coefficients across the Scenarios and assume a linear relationship between energy consumption and benefits. The unitary estimation for the resulted increase of the GDP due to the realized energy efficiency interventions was considered equal to 0.93 billion EUR/Mtoe. |
| Improved comfort level | Previous studies indicated that the energy efficiency investments considerably improve the households’ comfort levels. Other multiple benefits consist indicatively of the increased home durability, less maintenance, reduced noise levels, improved safety, such as fewer fires, reduced CO2 poisoning. A percentage of the cost savings is considered as the most effective metric, which can reach up to 25% of energy savings in the case that all multiple benefits are quantified. The respective value for the current tool was considered equal up to 10% of the achieved cost savings indicating a rather conservative estimate. The cost savings are resulted by the reduction of the operating, maintenance and fuel expenses. |
| Increased the building value | The increasing building value is also considered as another essential impact. Specifically, the increased value of the renovated buildings can be estimated as the ratio of the cost savings due to the reduction of the operating, maintenance and fuel expenses to the capital rate of the building. The cost savings were calculated for each building separately, while the capital rate was considered equal to 3% for the case of the households taking into consideration that the current analysis is performed within the framework of the Social CBA. Note: The triggered impact by the increased value of the renovated buildings was considered only in the Scenarios, which foresee the implementation of energy efficiency interventions in the building envelope, while the improved comfort levels were assessed in the case of the individual installation of heat pumps to avoid overlaps between the two types of externalities. |

Table 5 shows the types of external benefits and costs for each Scenario. For the external costs and benefits, the data is extracted from a report by Ecofys. Ecofys investigated the external costs (euro/MWH) per type of technology and the report was published by the EU commission. These external costs are presented below.

Table 4: external costs of using different types of energy efficient technologies

| Data | Oil boiler-existing | Oil boiler-new | NG boiler-existing | NG boiler-new | Coal boiler-existing | Biomass boiler-existing | Biomass boiler-new | HP-existing | HP-new | Electricity-current | PV-new | Solar-new |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| External cost (EUR/MWh) | 32 | 27.2 | 20 | 17.9 | 27.2 | 20 | 11.2 | 23.5 | 14.2 | 48.5 | 14.1 | 9.6 |

Table 5: Summary of the external costs and benefits for all the Scenarios

| Scenarios | External costs | External benefits |
| --- | --- | --- |
| Scenario 0 – higher energy efficient gas boiler | - Environmental costs triggered by the utilisation of the fossil gas boiler | - Avoided environmental costs due to substitution of the existing heating system by the fossil gas boiler - Multiple benefits of improving thermal comfort and reducing energy poverty |
| Scenario 1 – building envelope improvement | - Energy security costs triggered by the utilisation of the existing heating system despite the occurred reduction due to the building envelope improvement | - Reduced energy consumption due to improved building envelope - Increasing the house value |
| Scenario 2 – heat pump installations | - Environmental costs triggered by the utilisation of the heat pump | - Avoided environmental costs due to substitution of the existing heating systems by the heat pump - Multiple benefits of improving thermal comfort and reducing energy poverty - Macroeconomic impacts |
| Scenario 3 – photovoltaic solar panels | - Environmental costs triggered by the operation of photovoltaic systems | - Reduced environmental costs due to reduction of conventional electricity used from the grid - Multiple benefits of reducing energy poverty - Increasing the house value |
| Scenario 4 – Integrated retrofit package including the improvement of the building envelope and heat pump installation | - Environmental costs triggered by the utilisation of heat pump | - Avoided environmental costs due to reduced use of the existing heating systems and the operation of the heat pump - Increased house value - Macroeconomic impacts |
| Scenario 5– zero energy buildings | - Environmental costs triggered by the utilisation of heat pump - Environmental costs triggered by the operation of photovoltaic systems | - Avoided environmental costs due to reduced use of the existing heating systems and the operation of the heat pump - Increased house value - Reduced environmental costs due to reduction of electricity used from the grid - Macroeconomic impacts |

### Step 5: quantify the CBA and SCBA components

The data for the monetary costs is extracted from the Greenpeace study which was entitled “Cost-benefit analysis of RES and energy efficiency interventions in residential buildings” and published in 2022. The selected cost data within the context of the Greek study were verified by external experts. Then, the selected data were modified as to consider also other European countries with data both from other available studies (e.g., IRENA, 2022, Renewable solutions in end-uses: Heat pump costs and markets) and from the ECF study (IEECP, 2022, Study on the impacts of policies to decarbonize residential buildings on energy poverty in CEE/SEE and mitigation strategies). Table 6 summarizes the main categories of costs and benefits, which are quantified in the tool.

Regarding the fuel benefits and cost savings, a uniform and conservative estimation was decided for the delivered cost savings by the application of the net-metering schemes in different European countries (assuming 30% - 50% cost reduction customised per country) since it was possible to model all the implemented schemes so as to receive more accurate results.

Table 6: Quantified categories of costs and benefits in the CBA

| Cost | Description |  |
| --- | --- | --- |
| Operational and maintenance costs | - Operational and maintenance cost of the new energy efficient equipment/technology |  |
| Operational and maintenance benefits |  | - Avoided operational and maintenance cost of the existing technology for the average-income group of households |
| Fuel costs |  | - Fuel cost of the new energy efficient equipment/technology |
| Fuel benefits |  | - Avoided fuel cost of the existing equipment/technology |
| External costs |  | - Environmental and health cost of the new energy efficient equipment/technology |
| External benefits |  | - Avoided environmental and health cost of the existing equipment/technology - Macroeconomic effects due to the new energy efficient equipment/technology - Effects due to the improved comfort levels due to the new energy efficient equipment/technology - Increased market value of the renovated buildings |
| Capital costs |  | - Capital cost of the new energy efficient equipment/technology |

### Step 6: Calculate the evaluation indicators

Using the above-mentioned assumptions and collected data the differences in costs and benefits between baseline and the alternative Scenario are assessed according to the following equations:

Costt=   [Costt ]alternative - [Costt ]baseline 						(1)

Benefitt =  [Benefitt ]alternative - [Benefitt ]baseline				(2)

The following inputs are used in the calculations of the CBA:

- The energy efficient equipment/technology for average-income group of households is assessed within the alternative Scenario and the existing equipment/technology within the baseline Scenario

- The total cost and benefits of each year is the result of adding the value of all categories of costs and benefits, while the time frame of the analysis depicts the lifetime of the longest living asset.

- The discount rate is the parameter to estimate the value of future costs and benefits compared to present ones incorporating the social perspective on how future benefits and costs should be valued against present ones. A common discount factor equal to 3% has been selected to assess the economic and social effectiveness of the examined scenarios. The specification of the discount factor was done considering the opportunity cost of the households, which is lower than the selected value.

- The Net Present Value is calculated subtracting the expected costs from the incurred benefits discounted with a proper discount rate according to the following equation:

NPV=Benefitt-Costst(1+r)t

The results of the CBA are given using three different indicators (NPV, IRR, B/C).

Regarding the indicators of the analysis based on which the comparison is made

- Net Present Value (NPV): Expresses the net value (benefit or cost) resulting from discounting to the present the annual net cash flows (i.e., the cash balance) over the lifetime of an investment. If the NPV is positive (>0) the investment is approved, otherwise it is rejected.

- Internal Rate of Return (IRR): It expresses the discount rate at which the NPV breaks down and thus the profitability of the investment is evaluated by comparing the IRR with the discount rate. If the IRR is greater than the discount rate, the investment is approved, otherwise it is rejected.

- Benefit to cost ratio (B/C): B/C summarizes the overall relationship between the relative costs and benefits of a proposed project. If a project has a B/C greater than 1.0, the project is expected to deliver a positive net present value to its investors.

### Data collection

The data to be utilised in this tool can be mainly collected from the Eurostat repository and national Household Budget Surveys (HBSs).

### Brief explanations of different Scenarios

In the following sub-section, the different analysed scenarios will be portrayed and briefly explained. The subsidy rate is assumed to be zero for all Scenarios.

### Scenario 0: Country specific baseline Scenario

Firstly, a baseline scenario is performed per country to understand what the current proposed policies and directed investments regarding the investment on the supply side measures that differ per country would result in. Apart from the expected investments for improving the heating system, no new additional upgrades of the residential sector are foreseen in this Scenario. The type of heating systems differs per country and based on the availability of types of energy sources.

### Scenario 1: Energy upgrade of the building envelopes

The following scenario analyses a situation in which the available national budgets are devoted to the investment on the supply side measures for both space heating and cooling purposes and replacing existing building envelopes including window frames, doors, walls, roofs, and floor area of the buildings with more energy efficient ones. This would result in reduced energy costs for the households due to the better energy performance of the buildings. Additionally, due to the latter, it is assumed that no instalment costs related to the replacement of the existing heating systems would be foreseen, as this would not be needed given the improved general energy efficiency. For each country, the share of fuels used per end-use (i.e., space heating and cooling, domestic hot water, cooking and lighting appliances) and the price per fuel are considered to calculate the economic benefit related to such policy measures. Finally, the decreasing energy security of gas was considered by including an externality factor in the calculations.

### Scenario 2: Heat pump installations

In this Scenario, it is considered that the available national budgets for the development and expansion of the supply sides investments would be used to install heat pumps in the residential buildings. This would result in reduced energy costs for space heating and thus energy expenses in general. Similarly to Scenario 1, it was assumed that there would be no need to replace existing fossil gas boilers with higher energy efficient boilers since the old gas boilers will be replaced by the heat pump. Additionally, the same principle for calculating the economic benefits per country is employed.

### Scenario 3: Photovoltaic solar panel installation

Scenario 3 examines a situation in which the available budget for the supply side investments would be utilised for installing photovoltaic systems in the residential sector through energy offsetting schemes. This would once again reduce general energy expenses for the households and also enable them to offset the extra electricity produced. It must be noted that costs related to the upgrade of the electric network of the house, utility and electrician are not considered.

### Scenario 4: Integrated energy retrofitting including the energy upgrade of the building envelopes and installations of heat pumps

Scenario 4 evaluates the improvement of the building envelope conducted together with the installation of a heat pump. The logic behind the integrated renovations lies on the technical aspects of renovations. A highly insulated building is the requirement for the installation of heat pumps. The reason is that the heat pumps may provide lower temperature compared to the gas boilers or biomass boilers and therefore reducing the comfort level for the occupants. Therefore, it is highly recommended to improve the insulation of the buildings before the installation of heat pumps and Scenario 4 tests whether this retrofit package provides higher economic and social values for the households.

### Scenario 5: Performance of zero-energy buildings by combining all investments

The present scenario analyses a situation in which the available budget for the supply side investments would be employed to promote zero-energy buildings by combining a set of different investments. Namely, thermally insulating the exterior masonry, improving the cavity wall and solid insulation on walls, roof, floors, façade, replacing existing window frames with more energy efficient ones, installing heat pumps in residential buildings, and installations of the photovoltaic solar panels. This would result in a great reduction of energy consumption and expenses due to the improved efficiency. Once again, it is considered that no investment costs would be incurred related to the replacement of existing fossil gas boilers, as there would be no need for this. Additionally, the same principle for calculating the economic benefit per country resulting from the application of these measures is applied.

## Application of the tool

Hereby, illustrations will be shown as an example from the application of the tool in Romania, specifically for low-income households. Table 7 illustrates the typeof data needed as input for the tool. Table 8 shows additional inputs and related results from the application of the tool to perfrom a CBA of Scenario 1. Table 9 shows the results per year resulting from a CBA of Scenario 1 in Romania for low-income households. Table 10 and 11 illutsraste the same things but for the SCBA in Romania for low-income households.

Table 7: Input data needed.

| Romania | Total | Total | Heating | Cooling | DHW | Cooking | Electric appliances & Lighting |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Electricity | 1,830 | 16 | 42 | 35 | 1 | 1,736 |
|  | Heating oil | 67 | 0 |  | 14 | 53 |  |
|  | LPG | 465 | 1 |  | 97 | 367 |  |
|  | Natural gas | 3,955 | 2,315 |  | 899 | 740 |  |
|  | Solar thermal | 0 | 0 |  | 0 |  |  |
|  | Ambient heat | 0 | 0 |  |  |  |  |
|  | Biomass | 4,942 | 4,232 |  | 632 | 78 |  |
|  | District heating | 1,144 | 1,144 |  | 0 |  |  |
|  | Other | 66 | 46 |  | 15 | 5 |  |
|  | Total | 12,467 | 7,753 | 42 | 1,692 | 1,244 | 1,736 |

Table 8: Illustration of input and results from CBA for Scenario 1

Table 9: Illustration of results per year resulting from CBA for Scenario 1

| Year | O&M Cost | O&M Benefit | Fuel cost | Fuel benefit | Investment cost | Remaining value | Net cash flow |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  | 3,262,500,000 € |  | -3,262,500,000 € |
| 1 | 0 € | 0 € | 0 € | € 86,698,123 |  |  | € 86,698,123 |
| 2 | 0 € | 0 € | 0 € | € 86,698,123 |  |  | € 86,698,123 |
| 3 | 0 € | 0 € | 0 € | € 86,698,123 |  |  | € 86,698,123 |
| 4 | 0 € | 0 € | 0 € | € 86,698,123 |  |  | € 86,698,123 |
| 5 | 0 € | 0 € | 0 € | € 86,698,123 |  |  | € 86,698,123 |
| 6 | 0 € | 0 € | 0 € | € 86,698,123 |  |  | € 86,698,123 |
| 7 | 0 € | 0 € | 0 € | € 86,698,123 |  |  | € 86,698,123 |
| 8 | 0 € | 0 € | 0 € | € 86,698,123 |  |  | € 86,698,123 |
| 9 | 0 € | 0 € | 0 € | € 86,698,123 |  |  | € 86,698,123 |
| 10 | 0 € | 0 € | 0 € | € 86,698,123 |  |  | € 86,698,123 |
| 11 | 0 € | 0 € | 0 € | € 86,698,123 |  |  | € 86,698,123 |
| 12 | 0 € | 0 € | 0 € | € 86,698,123 |  |  | € 86,698,123 |
| 13 | 0 € | 0 € | 0 € | € 86,698,123 |  |  | € 86,698,123 |
| 14 | 0 € | 0 € | 0 € | € 86,698,123 |  |  | € 86,698,123 |
| 15 | 0 € | 0 € | 0 € | € 86,698,123 |  |  | € 86,698,123 |
| 16 | 0 € | 0 € | 0 € | € 86,698,123 |  |  | € 86,698,123 |
| 17 | 0 € | 0 € | 0 € | € 86,698,123 |  |  | € 86,698,123 |
| 18 | 0 € | 0 € | 0 € | € 86,698,123 |  |  | € 86,698,123 |
| 19 | 0 € | 0 € | 0 € | € 86,698,123 |  |  | € 86,698,123 |
| 20 | 0 € | 0 € | 0 € | € 86,698,123 |  |  | € 86,698,123 |
| 21 | 0 € | 0 € | 0 € | € 86,698,123 |  |  | € 86,698,123 |
| 22 | 0 € | 0 € | 0 € | € 86,698,123 |  |  | € 86,698,123 |
| 23 | 0 € | 0 € | 0 € | € 86,698,123 |  |  | € 86,698,123 |
| 24 | 0 € | 0 € | 0 € | € 86,698,123 |  |  | € 86,698,123 |
| 25 | 0 € | 0 € | 0 € | € 86,698,123 |  | € 435,000,000 | € 521,698,123 |
| Total | 0 € | 0 € | 0 € | € 1,909,366,256 | € 3,262,500,000 | € 430,693,069 | -€ 1,013,934,471 |

Table 10: Illustration of input and results from SCBA for Scenario 1

Table 11: Illustration of results per year resulting from SCBA for Scenario 1

| Year | O&M Cost | O&M Benefit | Fuel cost | Fuel benefit | External cost | External benefit | Investment cost | Remaining value | Net cash flow |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  |  | € 2,752,321,357 | € 2,741,596,639 |  | € 10,724,718 |
| 1 | € 0 | € 0 | € 0 | € 82,569,641 | € 0 | € 137,952,532 |  |  | € 220,522,172 |
| 2 | € 0 | € 0 | € 0 | € 82,569,641 | € 0 | € 137,952,532 |  |  | € 220,522,172 |
| 3 | € 0 | € 0 | € 0 | € 82,569,641 | € 0 | € 137,952,532 |  |  | € 220,522,172 |
| 4 | € 0 | € 0 | € 0 | € 82,569,641 | € 0 | € 137,952,532 |  |  | € 220,522,172 |
| 5 | € 0 | € 0 | € 0 | € 82,569,641 | € 0 | € 137,952,532 |  |  | € 220,522,172 |
| 6 | € 0 | € 0 | € 0 | € 82,569,641 | € 0 | € 137,952,532 |  |  | € 220,522,172 |
| 7 | € 0 | € 0 | € 0 | € 82,569,641 | € 0 | € 137,952,532 |  |  | € 220,522,172 |
| 8 | € 0 | € 0 | € 0 | € 82,569,641 | € 0 | € 137,952,532 |  |  | € 220,522,172 |
| 9 | € 0 | € 0 | € 0 | € 82,569,641 | € 0 | € 137,952,532 |  |  | € 220,522,172 |
| 10 | € 0 | € 0 | € 0 | € 82,569,641 | € 0 | € 137,952,532 |  |  | € 220,522,172 |
| 11 | € 0 | € 0 | € 0 | € 82,569,641 | € 0 | € 137,952,532 |  |  | € 220,522,172 |
| 12 | € 0 | € 0 | € 0 | € 82,569,641 | € 0 | € 137,952,532 |  |  | € 220,522,172 |
| 13 | € 0 | € 0 | € 0 | € 82,569,641 | € 0 | € 137,952,532 |  |  | € 220,522,172 |
| 14 | € 0 | € 0 | € 0 | € 82,569,641 | € 0 | € 137,952,532 |  |  | € 220,522,172 |
| 15 | € 0 | € 0 | € 0 | € 82,569,641 | € 0 | € 137,952,532 |  |  | € 220,522,172 |
| 16 | € 0 | € 0 | € 0 | € 82,569,641 | € 0 | € 137,952,532 |  |  | € 220,522,172 |
| 17 | € 0 | € 0 | € 0 | € 82,569,641 | € 0 | € 137,952,532 |  |  | € 220,522,172 |
| 18 | € 0 | € 0 | € 0 | € 82,569,641 | € 0 | € 137,952,532 |  |  | € 220,522,172 |
| 19 | € 0 | € 0 | € 0 | € 82,569,641 | € 0 | € 137,952,532 |  |  | € 220,522,172 |
| 20 | € 0 | € 0 | € 0 | € 82,569,641 | € 0 | € 137,952,532 |  |  | € 220,522,172 |
| 21 | € 0 | € 0 | € 0 | € 82,569,641 | € 0 | € 137,952,532 |  |  | € 220,522,172 |
| 22 | € 0 | € 0 | € 0 | € 82,569,641 | € 0 | € 137,952,532 |  |  | € 220,522,172 |
| 23 | € 0 | € 0 | € 0 | € 82,569,641 | € 0 | € 137,952,532 |  |  | € 220,522,172 |
| 24 | € 0 | € 0 | € 0 | € 82,569,641 | € 0 | € 137,952,532 |  |  | € 220,522,172 |
| 25 | € 0 | € 0 | € 0 | € 82,569,641 | € 0 | € 137,952,532 |  | € 435,000,000 | € 655,522,172 |
| Total | € 0 | € 0 | € 0 | € 1,818,444,053 | € 0 | € 5,790,471,439 | € 2,741,596,639 | € 430,693,069 | € 5,206,518,127 |

The basic principles for conducting a CBA as described in the “Methodology development for comprehensive Assessment” report of the JRC were incorporated into the presented methodology in Figure 1.

It should be highlighted that the proposed methodology was developed within the framework of PRODESA project to assess the energy efficiency interventions in public buildings.

Alberici et al., 2014. Subsidies and costs of EU energy. Ecofys by order European Commission.

Russell et al., 2015. Recognizing the Value of Energy Efficiency’s Multiple Benefits. Report IE1502.

https://ec.europa.eu/energy/content/final-report-ecofys_da

The environmental costs are much lower than the existing technologies. But they still have environmental costs, and we cannot neglect it. For example, for oil boiler is 32 EUR/MWh and for the new solar panel is 14.1 EUR/MWh.